# WELL APPLICATION CHECKLIST

(See Chapter 40E-3, FAC for the Rules on Well Construction)

## Septic Permit No. _______________ Property Tax No. (STRAP#)______________________________________

- All well applications must be filled out completely. If an item does not apply, please mark Not Apply.

- A site plan must accompany the well permit application with a map indicating the site address and directions to the property.    The well must be properly marked on the site plan showing all required setbacks. The setbacks are as follows:

Minimum of 25’ ft. from building foundation.

Domestic wells: minimum of 75’ft. from septic systems.

Irrigation wells: minimum of 75’ft. from septic systems.

Public supply wells minimum setbacks from septic systems:

100’ft. from septic systems where sewage flow rate is less than 2000 GPD.

200’ft. from septic systems where sewage flow rate is greater than 2000 GPD.

Minimum of 10’ft. from sewer lines.

3.	All septic systems or regional sewer lines (within 100’ft.) must be shown on the site plan including adjacent properties.

4.	The well application must be signed by both the well contractor and the property owner, or accompanied by agent authorization letter.

- Installation of new wells associated with a septic system must have the septic system permit number on the

well application.

- Rotary wells are to be grouted from bottom to top. Portland cement +5% bentonite, Neat cement +5% bentonite up to Type II.

- Cable tool wells are to be grouted bottom 5 ft, top 20 ft.

- Wells that are in conflict with automobile or road hazards may be cased at the ground level and identified by a traffic bearing valve box.

- Upon installation of a new well, prior existing wells being replaced must be immediately abandoned.   Exception: Unless placing the prior existing well into alternate service within 90 days, i.e., for irrigation. Existing wells can be used as monitoring wells, only when requested by SFWMD.

- Limited Use wells shall be equipped with a minimum six-foot square concrete apron four inches thick centered on the well casing.

- Well casings shall extend at least 18 ” above the ground surface (at least 12”inches above the concrete apron surface for Limited-Use wells) and one foot above the 100 year flood level.  The concrete well apron must be above the building finished floor level or the crown of the road, which ever is higher.

- Wells with a water usage of less than 100,000 gallons per day (except for domestic and fire wells) also need a SFWMD General Water Use Permit (permit duration is 20 years).

13.	The original (yellow) completion report must be submitted to the Hendry County Health Department. All well completion reports must be submitted to the HCHD within 30 days of the completion of the work as per rule 40E-3.411 (1) Florida Administrative Code, FAC.